# Water Resources Engineering Assignment Solutions

## Question 2: Statistical Analysis of Annual Peak Discharge

The dataset contains 16 years of annual peak discharge values (cumecs).

### 1) Data Preparation

Sorted values (ascending):

1557, 1625, 1913, 2157, 2405, 2602, 2662, 2934, 3120, 3655, 3670, 4005, 4621, 4630, 6216, 6403

### 2) Measures of Central Tendency

- Mean x:

x=∑xin=5417516=3385.94 cumecs

- Median (average of the 8th and 9th values):

Median=2934+31202=3027 cumecs

- Mode: No value repeats, so the dataset is amodal.

### 3) Measures of Dispersion

- Range:

Range=6403−1557=4846 cumecs

- Sample standard deviation:

s=∑(xi−x)2n−1≈1452.34 cumecs

- Coefficient of variation:

Cv=sx=1452.343385.94≈0.429 (42.9%)

### 4) Shape of the Distribution

- Skewness:

Cs≈0.81

Interpretation: Since Cs>0, the data is positively skewed (right-skewed), indicating a few high-flood years pull the mean above the median.

- Kurtosis:

Ck≈−0.12

Interpretation: The value is close to zero (near-normal) but slightly negative, indicating a slightly platykurtic distribution.

## Question 5: Fitting an Exponential Distribution to Rainfall Data

Given grouped rainfall observations over 365 days, the rainy-day class frequencies sum to:

90+49+34+17+13+5+3+2+1+1=215 days

So, 150 days are interpreted as zero-rainfall days.

### 1) Relative Frequency Histogram

For each rainfall class, relative frequency is:

Relative frequency=fi215

Example for the 0–10 mm class:

90215≈0.418

### 2) Exponential Model Fitting

The exponential probability density function is:

f(x)=λe−λx,  λ=1x

#### Step A: Estimate mean rainfall for rainy days

Using class midpoints mi:

x=∑(fimi)∑fi

x=(90⋅5)+(49⋅15)+(34⋅25)+⋯+(1⋅95)215≈4655215≈21.65 mm

#### Step B: Estimate parameter λ

λ=121.65≈0.0462 mm−1

### 3) Probability of Exceedance

For an exponential model:

P(X>x)=e−λx

For x=45 mm:

P(X>45)=e−(0.0462)(45)=e−2.079≈0.125

Therefore, the probability that a randomly selected rainy day exceeds 45 mm rainfall is approximately:

12.5%