# Estimated Water Consumption from Direct and Indirect Use

Student: [Your Name]

Course/Unit: [Course Title]

Institution: [University Name]

Date: 09 April 2026

# Abstract

This report estimates personal water consumption from direct household activities (showering, toilet flushing, car washing, and laundry) and indirect water use embedded in food choices (virtual water). Using standard engineering assumptions for flow rates, appliance water intensity, and published global-average water footprint factors for poultry-based meals, the results indicate that indirect (dietary) water use can exceed direct household use by an order of magnitude. A sensitivity range is presented, particularly for car washing method and shower flow rate, which are the largest uncertainties.

# 1. Introduction

Water consumption occurs in two major forms:

Direct water use: physically withdrawn and used at home (e.g., bathing, flushing).

Indirect water use (virtual water / water footprint): water used along supply chains to produce consumed goods, especially food.

For many individuals, diet-related indirect water dominates total water footprint, particularly when animal protein is frequently consumed.

# 2. Provided Personal Activity Data (User Inputs)

Showering: once per day

Eating: two meals per day, mainly fast foods (chicken)

Car washing: once per day

Laundry: once per week

Washroom visits: ~3 times per day (assumed 1 flush per visit)

# 3. Methodology and Assumptions

Because device efficiencies (low-flow showerhead, toilet flush volume, washing machine type, car wash method) were not specified, the report uses typical values and includes a sensitivity range where it matters most.

## 3.1  Direct Water Use Assumptions (Engineering Typical Values)

### A)  Showering

Typical shower flow rate: 9 L/min (standard showerhead)

Typical duration: 8 minutes/shower

Shower water per day:

Shower water = 9 × 8 = 72 L/day

### B)  Toilet Flushing

Typical flush volume: 6 L/flush (modern dual/standard efficiency)

Flushes per day: 3 (one per washroom visit)

Toilet water per day:

Toilet water = 6 × 3 = 18 L/day

### C)  Car Washing (Largest Uncertainty)

Car washing water depends strongly on method. Typical ranges are shown below, and a base-case of 350 L/wash (hose wash mid-range) is used because daily car washing most commonly implies open-hose use.

| Method | Water Use (L/wash) |
| --- | --- |
| Bucket wash | ~50–80 |
| Hose wash (uncontrolled) | ~300–500 ← base case mid: 350 |
| Commercial wash (partial recycling) | ~120–200 |

### D)  Laundry

Typical washing machine water: ~100 L/load

Frequency: 1 load/week

Average laundry water per day:

Laundry water per day = 100 L7 ≈ 14.3 L/day

## 3.2  Indirect Water Use (Food / Virtual Water) Assumptions

The diet described is two meals per day of mainly fast-food chicken. Indirect water depends on portion size and the embedded water in chicken plus sides (bread, fries, sauces).

Key factor (global average, water footprint literature):

Chicken meat water footprint ≈ 4,300 L/kg (global average; varies by production system and region)

Portion assumption:

Chicken per meal ≈ 150 g (0.15 kg) of chicken content (e.g., fillet, burger patty equivalent)

Two meals/day → chicken mass per day = 0.30 kg/day

Chicken-only indirect water estimate:

Chicken virtual water = 0.30 kg/day × 4 300 L/kg = 1 290 L/day

For a full fast-food meal (including bun, fries, condiments) a conservative meal multiplier is applied, yielding approximately 1,800 L/meal:

Meal virtual water = 1 800 L/meal × 2 meals/day = 3 600 L/day

Note: This 'whole meal' factor is intentionally conservative and should be refined if exact meal composition and portion sizes are known.

# 4. Calculations and Results

## 4.1  Direct Water Use (Base Case)

| Activity | Assumption | Calculation | Estimated Use (L/day) |
| --- | --- | --- | --- |
| Shower | 9 L/min, 8 min | 9 × 8 | 72 |
| Toilet flushing | 6 L/flush, 3/day | 6 × 3 | 18 |
| Car wash | Hose wash mid | — | 350 |
| Laundry | 100 L/week | 100 ÷ 7 | 14 |
| Total |  |  | 454 |

Converting to weekly and annual figures:

454 L/day × 7 = 3 178 L/week ≈ 3.18 m³/week

454 L/day × 365 = 165 710 L/year ≈ 165.7 m³/year

## 4.2  Indirect Water Use (Food / Virtual Water, Base Case)

| Activity | Assumption | Calculation | Estimated Use (L/day) |
| --- | --- | --- | --- |
| Fast-food meals (chicken-based) | 1 800 L/meal | 1 800 × 2 | 3 600 |

Converting to weekly and annual figures:

3 600 L/day × 7 = 25 200 L/week = 25.2 m³/week

3 600 L/day × 365 = 1 314 000 L/year = 1 314 m³/year

## 4.3  Combined Total Water Footprint (Base Case)

| Category | L/day | L/week | L/year |
| --- | --- | --- | --- |
| Direct | 454 | 3 178 | 165 710 |
| Indirect (food) | 3 600 | 25 200 | 1 314 000 |
| Total | 4 054 | 28 378 | 1 479 710 |

Interpretation: Under base-case assumptions, indirect (dietary) water represents approximately 89% of the total daily water footprint.

Total = Direct + Indirect = 454 + 3 600 = 4 054 L/day

# 5. Sensitivity Analysis (Uncertainty Ranges)

Two assumptions can significantly shift results: car washing method and shower flow rate/duration.

## 5.1  Car Wash Sensitivity (L/day)

Bucket wash: ~60 L/day

Commercial wash: ~150 L/day

Hose wash (uncontrolled): ~350 L/day (base case) up to ~500 L/day

This single variable can change direct water use by several hundred litres per day.

## 5.2  Shower Sensitivity (L/day)

Low-flow showerhead (~6 L/min) × 8 min = 48 L/day

Standard (9 L/min) × 8 min = 72 L/day  ← base case

Longer shower (9 L/min × 12 min) = 108 L/day

## 5.3  Range Summary (Direct Water Only)

| Scenario | Composition | Total Direct (L/day) |
| --- | --- | --- |
| Low case | 48 (shower) + 18 (toilet) + 60 (bucket wash) + 14 (laundry) | ~140 |
| High case | 108 (shower) + 18 (toilet) + 500 (hose wash) + 14 (laundry) | ~640 |

Even at the low end, if meals remain chicken fast-food based, indirect water still dominates total consumption.

# 6. Discussion

The calculations reveal a clear hierarchy of water impacts:

Diet choice (indirect water) is the largest contributor, especially with animal-protein-heavy fast foods.

Daily car washing can become the dominant direct use if performed with an open hose.

Showering and toilet flushing are moderate contributors; savings are achievable through efficient fixtures.

Laundry is relatively small when averaged to a daily figure at once per week.

The indirect water estimates are global-average based and can vary significantly due to feed sources, farming practices, climate, and supply-chain efficiency. For a more locally accurate estimate, Uganda- or region-specific footprint data should be used where available.

# 7. Recommendations (Water Reduction Options)

## 7.1  Direct Water Savings

Switch from open-hose to bucket car washing or use a shut-off nozzle: potential saving ~200–400 L/day.

Install a low-flow showerhead and reduce shower duration by 2–4 minutes: save ~18–36 L/day.

Use a dual-flush toilet or adjust cistern volume: save ~3–10 L/day depending on the current toilet.

## 7.2  Indirect (Diet) Water Savings

Replace one chicken-based fast-food meal per day with a more plant-based option. Even partial shifts can reduce virtual water use by hundreds to thousands of litres per day depending on replacement foods.

# 8. Conclusion

Based on typical engineering values and conservative food footprint assumptions, the estimated daily water footprint is:

Direct use: ~454 L/day  (range ~140–640 L/day)

Indirect food-related use: ~3,600 L/day  (dominant share)

Combined: ~4,054 L/day ≈ 4.05 m³/day

The most effective reduction strategies are (1) reducing the water intensity of daily car washing, and (2) moderating the frequency and portioning of chicken-based fast-food meals.

# References

Water Footprint Network (WFN): food and product water footprint concepts and data. Available at: waterfootprint.org

Food and Agriculture Organisation (FAO): resources on livestock and water use. Available at: fao.org

US EPA WaterSense: household fixture flow rates and conservation. Available at: epa.gov/watersense

Academic literature on life-cycle water use in poultry production (peer-reviewed LCA studies).

(Optional) Local utility or national water authority publications for Kampala/Uganda context.

# Appendix A: Quick Formula Summary

Shower (L/day):  Q_{shower} = Flow rate (L/min) × Duration (min/day)

Toilet (L/day):  Q_{toilet} = Flush volume (L/flush) × Flushes per day

Laundry (L/day):  Q_{laundry} = Litres per load7

Chicken virtual water (L/day):  VW_{chicken} = Chicken mass (kg/day) × 4 300 (L/kg)

Meal-based virtual water (L/day):  VW_{meal} = Water per meal (L/meal) × Meals per day